There are several tools available for you to “view” an audio signal.  One of the most common is a VU meter – either an actual meter movement (pretty rare these days) or an LED display.  In either case, the measurement is of the instantaneous total audio signal level.  You are viewing the audio voltage over time.  This is the same measurement that is performed with an oscilloscope only with a significantly different presentation.

Another tool is the spectrum analyzer.  This instrument allows you to view the frequency components and their relative levels.  Although you can get audio Spectrum Analyzer applications for your PC, tablet or Smart Phone, sometimes it is preferable to have a device dedicated for this purpose.

Using a “real” spectrum analyzer can be very helpful if you need to do a detailed analysis of the frequency response of a system.  However, during normal usage of an audio system, such as monitoring the output of a PA system console, it can show too much data to be useful.

This article describes an Audio Spectrum Display that can be configured for ISO standard frequencies – those that typically are used for graphics equalizers.  This system uses sets of ten LEDs to show the relative amplitudes of bands of frequencies.  The system can be implemented with essentially any number of frequency bands displayed.  The standards currently being used are 10, 15 and 31.  Most of the discussion in this article refers to the unit I built which is 10 bands.

Circuit Descriptions

There are essentially three circuits in the display assembly: power supply, preamp and filters.  Each is described here along with some of the reasoning for the design decisions.  An overall block diagram is shown in Fig. 1.

Power Supply – Fig. 2

There are two voltages required by the system: +12V and +22V.  The power supply circuit can be powered by a single +12V to +18V, 100ma unregulated DC wall wart.  If you use a regulated unit, it should be at least 13V due to the protection diode in series with the input.  The +12V circuit uses a 12V, low drop-out regulator which requires only 12.5V for operation.  Its maximum allowable input is 20V, so you should measure the unloaded output voltage of your wall wart to ensure you do not exceed this value.  The 22V is derived from a boost mode switching regulator and will work with any voltage from 8V to 20V.  If you already have a +12V regulated power supply, you can use it by simply shorting the input and output pins of U101 – the 12V regulator.  You can use up to a +16V supply since the maximum Vdd for the opamps is 16V.  However, if you do use more than 12V you may want to change the resistive divider for the bias voltage (R212 and R214) in order to keep it at 6V.

In theory the circuits should work fine with an unregulated power supply.  However, I prefer to use simple regulators for most of my circuits.  I also wanted to develop a fixed and known voltage for the LM3915 references.  For circuits that use a single LM3915 I typically use its internal reference feature.  Since this system uses at least ten LM3915s I wanted the same reference for all of them.  There is no simple way to use the reference from one of them for the remaining ones and keep all the circuits identical.  See Figure 3 for the 6V reference using U201.1.

Both the preamp and the power supply are on a single board.  The layout is such that the ground for the +12V regulator and the ground for the +22V switching regulator are kept separate and are connected together by a single jumper – H102.  I did this in order to keep the LED current and the switching regulator noise out of the analog circuits as much as possible.  Make sure you insert a jumper wire between the two points or the circuit will not work properly.

The Power Supply/PreAmp board has three six-pin headers on it, all connected in parallel.  These allow you to connect up to three Filter boards without resorting to splicing wires.  There are also two pads in the Power Supply circuit which allow you to insert your own power supply voltages if you wish – H1 for Vdd and H2 for HiV.

Most of the passive components in this system are surface mount 0805.  However, the three large capacitors in this circuit are thru-hole because the surface mount packages for the equivalent parts are very hard to solder manually.

If you are going to build either the 15 band or 31 band units, R101 and R102 may need to be decreased in value.  I put two resistors in parallel here in order to allow the 0805 parts to handle the higher current which may be required.  The formula in the regulator specifications for the resistor evaluates to approximately: R = 0.18/I.  For instance, if each string in a ten band display draws 5ma for a total of 50ma, then R101 & R102 in parallel should be about 3.6 ohms.

PreAmp – Fig. 3

The first stage is a differential amplifier with a gain of two.  The input can be balanced or unbalanced and the load is approximately 10K.  The system was essentially designed to not load the applied signal – assuming it is low impedance.  I selected the gain of two so that a 0dBm signal will allow an almost full scale indication with the second stage gain is set to one.

The second stage has three selectable gain values: 0dB (x1), 10dB (x3.16) and 20dB (x10).  For circuit simplicity, the center off position of the switch is the highest gain.  The output of the second stage feeds a fixed resistor and potentiometers so that the output of the filters can be calibrated to display one LED below full scale when the potentiometer (R213) is fully clockwise and a signal of 0dBm (0.775Vrms into 600 ohms) is applied.  Stage three is a simple follower which can drive all of the filters in parallel.

The value of resistor R210 and trimpot R215 are calculated based on the Q of the filter circuits.  For the ten-band unit the required Q (and the gain) is 2.5.  Ignoring trimpot R215 for the moment, this requires a series resistor of 5.71K with the closest 1% value (lower than 5.71K) being 5.62K.  The 15-band display requires a Q of 4 (R = 15K) while the 31-band display requires a Q of 8.7 (R = 45K). (See the description of the filter circuit for an explanation of the Q values.) These resistor values ensure that a 0dBm signal applied to the preamp will yield an almost full scale indication.

The purpose of R215 is to allow you to calibrate the gain of the system if you wish.  If not, you can simply short out pins 2 and 3 and use the suggested fixed resistor.  If you do use R215 you may still want to use R210 in conjunction with it.  Suggested values are as follows...

for the 10-band unit: R210 = 2K, R215 = 5K

for the 15-band unit: R210 = 10K, R215 = 10K

for the 31-band unit: R210 = 30K, R215 = 25K

If you want a larger range of adjustment, you can short R210 and simply have R215 large enough to cover the range you want.

If you have a need for a different range of input levels, there are several easy ways you can change the circuit.  The main criteria you must keep in mind is that it takes 7Vp-p into the rectifier circuit (U1.3) on the filter board for the display to show full scale.  That is about 2.8Vp-p into the filter of a ten-band unit.  The easiest way to allow a higher input level is to reduce the gain of the input stage by reducing the values of R204 and R211.  Another possibility is to change the resistors on SW210 – R201, R202, R207.

Since a single supply is being used, one section of the op-amp (U201.1) is used to develop a buffered bias voltage centered on the power supply.  This bias is used to create a virtual ground so that the signals become centered on this voltage.  This is why capacitive coupling is being used on the input and output of the preamp.  The 10uf capacitors have a reactance of 513 ohms at 31Hz.  This equates to about 0.5dB drop in signal which I feel is quite acceptable.  I would rather have used a transformer but one that has good low frequency response is quite expensive.

Filter – Fig. 4

The design of the filter circuits is derived from an article I found on the web: http://www.vidyasagar.ac.in/journal/maths/Vol11/JPS11-16.pdf.  The circuit described in the article uses two op-amps to create a high Q band pass filter.  The Q is controlled by the ratio of two resistors (R6 and R7).  In this particular circuit the value of Q is also the gain which, in my opinion, is its main disadvantage.  One of the nicer features of this filter design is that the three pairs of resistors and capacitors which determine the frequency have the same value. These two features are not always the case with many other filter configurations.

I determined the Q I needed by simulating the circuit using LTspice IV.  This is a free circuit simulator available from Linear Technology Corporation at: http://www.linear.com/designtools/software/#LTspice.  The simulation uses component values which yield a nominal frequency of 1KHz in order to determine what Q was needed so that the two “adjacent” frequencies were down about 12dB.  The value of 12dB is not a standard, it is simply what I chose to use.  See Photo 1 for an LTspice screen shot of the 1kHz filter.  I have included the LTspice file (Filter LTspice.asc) with the other files for this project.

The adjacent frequencies are determined by the number of discrete frequencies in the display.  The unit I built has ten bands centered on these frequencies: 31.25, 62.5, 125, 250, 500, 1000, 2000, 4000, 8000 and 16000.  You can see that each frequency is double the preceding one.  It is also possible to build 15-band and 31-band displays with these circuits.  There is a spreadsheet (Frequencies.ods) which shows the frequencies and suggested component values for each of the three display variations.  The frequencies I chose were determined by searching the web for Graphic Equalizers.

The LM3915 has ten comparators with 3dB spacing.  There are two other devices similar to the 3915 but with different spacing for the comparators – the LM3914 is linear while the LM3916 spacing is similar to the labeling of a VU meter.  If you prefer that the display be similar to that of a VU meter then all you have to do is use LM3916s instead of the LM3915s.  Each of the ten outputs is capable of driving an LED with up to 30ma.

This circuit has the LEDs for each band essentially in series.  The LM3915 has two modes of operation relative to the LEDs - bar and dot.  For most audio displays, the bar mode is preferred.  This circuit uses the dot mode, but with the LEDs in series to simulate bar mode.  In some designs, each LED is connected to the same low voltage power supply as the IC.  In that case each LED will draw its own current from its 3915 driver.  Even with only 5ma through each LED the total current, at full scale, would be 50ma.  Since this system has ten sets of LEDs, operating them could require up to 500ma (for the ten band unit) from the power supply depending on the audio signal being monitored.  This would also require that the PCB handle the high current such that it does not interfere with the analog circuitry.  I chose to use the series mode so that the same current (approximately 5ma) flows through each of the LEDs for each of the 3915s.  In order to do this, the LED supply must be able to handle the ten LEDs in series.  This can easily be 20V and is why I have the 22V boost regulator for the LED supply.  The LM3915 upper voltage limit is 25V which means that most blue or white LEDs will not work in this configuration.

The LEDs I used are high brightness with a nominal output of 2600mcd at 20ma and cost about $.18 each.  I originally operated the LEDs with 10ma but found that the display was too bright.  I changed R4 from my original value of 1.5k ohms to 3.3k ohms which runs them at about 4ma and is still is plenty bright!  Since the light intensity is proportional to the current, I estimate they are outputting about 520mcd.  The spacing on the PCB is such that you can use T 1¾ LEDs, some of which are about half the cost of the ones I used.  Photo 2 shows the unit monitoring some music.  You can see that the LEDs are still so bright that their light is somewhat diffused by the front cover of my enclosure.  I had to manually adjust my camera shutter speed in order to keep it from adding to the diffusion but it also makes the enclosure darker than it really is.

Mounting the LEDs to the circuit board was one of the more tedious operations of the assembly – especially since there are at least 100 of them!  If you use the LEDs I used and mount them so that do not protrude through the front panel, then you will probably want 0.4” spacers.

I use the first filter circuit, with a few minor component changes (see Fig. 5) as a display with no filtering.  Obviously, this circuit is more complex than is required for its function, but being able to use the existing PCB was too much of an advantage to waste.  You can compare the two schematics to see the differences.  Other than eliminating the frequency dependent components, the only changes are in the resistor values and in shorting out C3. The BOM file does not list these components.

Circuit Implementation and Construction

During the design phase of this project I developed several physical iterations of the circuit.  In general the basic schematic did not change.  The first design had each filter on a separate circuit board of 1.3” by 2.1”.  Each board would be at a right angle to the display surface with a board-to-board connector between each one.  The main advantage of this construction is that you can easily implement however many bands you want by simply changing the number of boards in your system.  Two disadvantages are (1) physically attaching the boards to an enclosure and (2) the cost of the board-to-board connectors.  Also, the total area of the boards is somewhat greater due in part to the connector.

I finally decided to implement several filter circuits on one board.  I designed boards with four, five and six circuits each but finally decided on the board with four circuits.  The main reason for this selection was the cost of the PCBs.  The manufacturer I have been using lately is OSH Park (http://www.oshpark.com/) which sells 2 layer boards at $5 per square inch (3 copies of the board are included in that price).  Since I was going to get three boards I decided to use the board with four filters to implement my ten-band display.  The board has a line in the silk screen that you can use as a guide to cut off the last filter circuit.  I did this so that the board can be shortened if desired to fit in an enclosure.

S1 through S8 in Figure 1 are hand-assembled connectors which mate to six pin headers on the circuit boards.  I have been using these connectors for quite a few years as the main method of I/O for my PCBs.  The lowest cost parts I have found are from Jameco and are listed in the BOM.  When purchased in quantity, enough for 100 connections, the additional cost is approximately $.18 per connection.  If you purchase just enough for this project, the cost is approximately $.27 per connection.  If you want to save a few dollars, you can simply solder connecting wires directly to the circuit boards.  In order to use these connectors you will need to cut them to size.  The pin headers can be cut to size without loosing any pins; however, when cutting the housings you will loose one pin per connector.  When I first started using these connectors I used a small pair of pliers to crimp the wires to the socket pins.  I have since obtained a crimp tool which makes it much easier.

The Block Diagram (Fig. 1) shows both 1/4” phone and XLR connectors.  I have installed both in order to make the system easier to use.  You can use whatever connectors suit your particular application.  If you choose to use XLRs, note that the pin connections are in reverse order between them and S2.  This is because S2 pin 3 is ground whereas pin 1 on an XLR is ground.

The Bill of Materials (BOM) for the system includes all the parts, except for some mounting hardware, used to build a ten-band display as described in this article.  The frequency specific parts are indicated as such.  The BOM includes the vendor part numbers.  I purchase most of my parts from DigiKey since they have free shipping if a check is included with the order.  If you use this service be sure to include whatever sales taxes are applicable.

The mounting holes in the boards are for 4-40 bolts.  You will need about 15 nuts and bolts.  You will also want some spacers for the bolts in order to keep the PCBs off the surface of the enclosure.  The length of the bolts and standoffs will depend on how you decide to mount the PCBs to the enclosure.

I found another source for the LM3915's that was significantly less expensive than DigiKey – Tayda Electronics (http://www.taydaelectronics.com/).  I checked Ebay for these ICs and found them also available from other sources; however,  I have no idea whether or not those listed are actual National Semiconductor parts.

The enclosure I selected is transparent blue.  I picked this one because I did not want to drill 110 holes in the front panel!  The LEDs show quite nicely through the front of the enclosure.  They are actually still a bit too bright.  If I build another unit I will probably use standard brightness LEDs!  Also, the box is just wide enough to handle the 11 columns of LEDs so I mounted the controls of the Power/PreAmp board on the side.

Photo 3 is a picture of the assembled unit.  Note that there are only the two ¼” connectors in the picture.  I had not mounted the XLRs when the picture was taken.  You will also see that the power indicator LED is hand-wired onto the PCB.  The circuit board on both my web site and the audioXpress web site has been updated to include it.  Also, the filter PCB on my unit has an LED spacing of 0.2” which works fine for the LEDs I used.  The PCB on both web sites has been updated with larger spacing in order to accommodate larger LEDs.

All the work files are available on my web site, http://www.qsl.net/k3pto/, as well as from audioXpress.  I use the program DipTrace, www.diptrace.com, for all my schematics and PCB designs.  A free version of the software is available from their website.

Conclusions and Further Thoughts

I have connected the unit to the audio output of my PC at home.  It is interesting to watch the display while listening to different types of music.  However, I find that I now wish I had built a unit with more bands.

As I mentioned earlier in this article, the current in the high brightness LEDs is a bit too high.  The LM3915s have a minimum output current which is still higher than I like for these LEDs.  I have since implemented a dimming circuit which should work fine for standard brightness LEDs.  This circuit is NOT the one on the OshPark website since I have not made any of the new PCBs.  Except for the two minor modifications mentioned above, I wanted the PCBs on the site to be exactly the ones I built for this article.

I have also designed a new set of display and filter PCBs, based on the same schematics, that enable a narrower assembly.  This will allow a 31 band display to be implemented within a 13 inch width.  It also has the dimming circuit mentioned above.  If you are interested in either of these two modifications please contact me directly at k3pto@arrl.net.

Figure 1: Block diagram

Figure 2: Power Supply

Figure 3: PreAmp

Figure 4: Filter

Figure 5: No Filter

Photo 1: LTspice analysis of 1kHz filter with Q = 2.5

Photo 2: Monitoring some music

Photo 3: Assembled display