Parts List-PA Board

Ref. Des.			Description			Vendor		P/N

C1,C3,C4,C7		100 uF  Al. Electrolytic		Panasonic	ECA-1HM101

C10,C11

C2,C6			330 pF Ceramic			AVX		SR151A331KAR

C5			100 pF Ceramic			AVX		SR151A101KAR

C8,C9,C12		100 nF					Epcos		B32520C1104J

C13			220 uF/25-v Al. Elect.			Panasonic	ECA-1EM221

CR1			1N5232B, 5.6-volt zener		Fairchild	1N5232B

CR2,CR3		1N4148 diode				Fairchild	1N4148B

CR4,CR5		Green T1 ¼ LED			Rohm		SLR-56MC3F

J1			RCA/Phono Jack			CUI		RCJ-011

L1			14 turns #18 enamel wire on R37

Q1,Q2,Q8,Q14	2N3906				Fairchild	2N3906

Q19

Q3,Q4,Q7		2N5401				Fairchild	2N5401

Q5,Q6,Q9		2N5550				Fairchild	2N5550

Q10,Q12,Q16		2SC4793				Toshiba	2SC4793

Q11,Q20		2N3904				Fairchild	2N3904

Q13,Q17		2SA1837				Toshiba	2SA1837

Q15			2SA1943				Toshiba	2SA1943

Q18			2SC5200				Toshiba	2SC5200

R1,R9,R31,42		1 k-ohm, 1/4-watt carbon film

R2,R3,R4,R5		100 ohm, 1/4-watt carbon film

R6,R17,R18

R20,R21,R24

R26,R38,R39

R7,R8,R28,R30	10 k-ohm, 1/4-watt carbon film

R32,R33,R34

R40,R41

R10			4.7 k-ohm, 1/4-watt carbon film

R11			22 ohm, 1/4-watt carbon film

R12			2 k-ohm potentiometer		Bourns		3386F-1-202LF

R13			1.5 k-ohm, 1/4-watt carbon film

R14,R15		2.2 k-ohm, 1/4-watt carbon film

R16			47 ohm, 1/4-watt carbon film

R19,R22		0.5 ohm, 5-watt wirewound		Vishay		CW005R5000J

R23,R25,R29		330 ohm, 1/4-watt carbon film

R27			5.0 ohm, 3-watt wirewound		Vishay		ALSR035R000J

R35,R36		10 ohm, 1/4-watt carbon film

R37			5 ohm, 5-watt wirewound		Vishay		ALSR055R000J

TO-220 Heatsinks for Q10,Q13	Wakefield	274-2AB

Notes:

All parts sourced from Digi-Key.

¼-watt resistors are 5% carbon film and can be sourced from multiple vendors.

Although R19, R22, and R37 may be sourced from other vendors and similar parts used, be careful of the diameters of the parts as R37 with L1 wound around it must lie next to R22, preferably without touching.

Attach Q12, Q16, and Q17 to common aluminum or copper bar stock to keep all at same temperature.

The small signal transistors, Q1, Q2, etc. may be sourced from alternate manufacturers such as ON Semiconductor or NXP.

Use a heatsink on Q10 and Q13; insulating hardware is not necessary as long as the tab and heatsink don’t touch ground (this shouldn’t be an issue).

Use thermal compound on Q10 and Q13 and on Q12, Q16, and Q17.

Q15 and Q18 are mounted on a large finned heatsink from Par-metal with thermal insulators and thermal compound. When I bought the parts from Digi-Key, the mica insulators were included. Warning: beware of counterfeits! Digi-Key is a reliable distributor and is recommended.