Parts List-Power Supply

Ref. Des.			Description			Vendor		P/N

BR1			25 A, 400 V bridge rect.		EIC Semi.	BR2504

F1			4A, 250 slow blow fuse

F2,F3,F4,F5		5A automotive fuse

C1,C2,C3,C4		10,000 uF/50-v alum elect		Panasonic	ECO-S1HA103DA

R1			56-ohms, 2-watt 			Yageo		FMP200JR-52-56R

R2,R3			1.5 k-ohm, 2-watt 			Yageo		FMP200JR-52-1K5

S1			SPST

T1			115/230 v to dual 30-v transformer	Avel		Y236802

Notes:

The bridge rectifier (BR1) I used may be hard to source. Parts Express 050-060 should work as well.

Other vendors’ parts may be used for the filter capacitors, even ones with screw terminals if that makes assembly easier. The 50-volt rating can be increased; nominal voltage at Vcc is approx. 42 volts.

S1 was an illuminated switch that I had on hand. Any switch rated for the appropriate voltage and current should work fine. Note that there is significant inrush current on power-on; a fairly high current rating is recommended.

R1 can be 47 ohms if 56 ohms is hard to source. A different composition or higher wattage rating may also be used. R2 and R3 may also use a different composition or higher wattage rating. The value is also not critical although if the value is changed, the power dissipation will change as well and this should be taken into account.

T1 is a 500 VA transformer. A 250 VA transformer may work for most applications and would have less turn-on inrush. Given the normal power levels I use, I should have used a smaller transformer. The Avel part number for the 250 VA part is Y236653 and is available from parts express.

To minimize power rail ripple caused by capacitor charging currents, attach the wires that go from the power supply to the PA boards as close to the capacitor terminals as practical.