Parts List-Speaker Protection Board

Ref. Des.			Description			Vendor		P/N

C1,C2			10 uF/63-v film cap			EPCOS	B32522C106K

C3			47 uF/35-v alum. Elect.		Panasonic	EEU-FC1V470

C4			10 uF/35-v alum. Elect.		Nichicon	UES1V100MEM

CR1			1N4750A, 27-volt zener		Fairchild	1N4750ATR

CR2,CR3,CR4,	1N4148				Fairchild	1N4148B

CR5,CR6,CR7,

CR10

CR8,CR9		1N4001				Fairchild	1N4001

Q1			TIP122 				Fairchild	TIP122

Q3,Q4,Q6,Q7		2N3904				Fairchild	2N3904

Q5			2N3906				Fairchild	2N3906

Q8			TIP47					Fairchild	TIP47

R1,R4,R5,R9		10 k-ohm, 1/4-watt carbon film

R8

R2,R3,R7		47 k-ohm, 1/4-watt carbon film

R6,R10,R17		100 k-ohm, 1/4-watt carbon film

R11,R12		33 k-ohm, 1/4-watt carbon film

R13,R14		1 k-ohm, 1/4-watt carbon film

R15			22 k-ohm, 1/4-watt carbon film

R16			270 k-ohm, 1/4-watt carbon film

RLY1,RLY2		SPDT relay, 12-v, 40 A		Picker		PC792A-1C-P-12S-X

TO-220 Heatsink for Q1		Wakefield	274-2AB

Notes:

All parts sourced from Digi-Key except RLY1 and RLY2.

The relays are automotive types, 12-volt coil with 40 A contact rating. The Picker relay used here should be available from Omni Pro (800) 926-8926. A TE Connectivity Relay 1432868-1 may work as well as many other similar automotive relays. The pinout is very common, but the PC-mount pins may not be.

¼-watt resistors are 5% carbon film and can be sourced from multiple vendors.

The transistors may be sourced from alternate manufacturers such as ON Semiconductor or NXP.

Use a heatsink and thermal compound on Q1 and note that the tab must be insulated from ground. It is easiest to apply Q1 to the heatsink with thermal compound and then place an insulating sheet e.g. mica, paper, between the sink/transistor and the board. Use an insulating washer on the mounting screw.

If the normally-open inputs on the board are not going to be used, as is the case with my design, place a wire short between the two NO pads.

Observe the 35-volt limit on Vcc to prevent overheating of Q1. For operation above 35 volts a resistor of approximately 8 ohms per volt above 35 volts should be connected in series with the Vcc input.